Computer Emergency Response Team of Mauritius

A department under the Ministry of Information Technology, Communication and Innovation

CERT-MU Security Advisory

CERT-MU Advisories AD-2026-02

Cisco ACI Multi-Site CloudSec Encryption Information Disclosure Vulnerability

Date of issue: 24 April 2026

Severity Rating: High

Systems Affected:

- This vulnerability affects Cisco Nexus 9000 Series Fabric Switches in ACI mode that are running releases 14.0 and later if they are part of a Multi-Site topology and have the CloudSec encryption feature enabled.

CVE Information:

CVE-2023-20185

Description

A vulnerability has been identified in the Cisco ACI Multi-Site CloudSec encryption feature of Cisco Nexus 9000 Series Fabric Switches in ACI mode and this could allow an unauthenticated, remote attacker to read or modify intersite encrypted traffic.

This vulnerability is caused due to an issue with the implementation of the ciphers that are used by the CloudSec encryption feature on affected switches. An attacker with an on-path position between the ACI sites could exploit this vulnerability by intercepting intersite encrypted traffic and using cryptanalytic techniques to break the encryption. A successful exploit could allow the attacker to read or modify the traffic that is transmitted between the sites.

Mitigation

It is to be noted that Cisco has not released and will not release software updates to address the vulnerability that is described in this advisory.

The affected Cisco ACI Multi-Site CloudSec encryption feature has been deprecated in Cisco NDO Release 4.2(1d) and has been removed in Cisco ACI Release 6.2(2e).

Users who are using the Cisco ACI Multi-Site CloudSec encryption feature for Cisco Nexus 9332C and Nexus 9364C Switches and Cisco Nexus N9K-X9736C-FX Line Cards are advised to disable this feature and to contact their support organisation to evaluate alternative options, such as performing encryption on the underlying site-to-site connections. There are no alternatives that provide full encryption for data in transit between sites on current ACI spine switches hardware.

References

https://sec.cloudapps.cisco.com/security/center/content/CiscoSecurityAdvisory/cisco-sa-aci-cloudsec-enc-Vs5Wn2sX

Postal address 
Computer Emergency Response Team of Mauritius (CERT-MU) 
Ministry of Information Technology, Communication and Innovation

3rd Floor, Wing A,

Shri Atal Bihari Vajpayee Tower,

Cybercity Ebene,